David Campion, Michael Flynn, Nathan Field & Nikita Fedans

A 2FA Security System for securing single-entry rooms

DMN sYSTEMS

DMN Alarms

DMN Alarms

David Campion, Michael Flynn, Nathan Field & Nikita Fedans

Table of Contents

Introduction	2

Hardware	3

Circuit Diagrams	3

Hardware Requirements:	4

Data, Data Storage and Data Processing	5

What Data is gathered	5

How it will be stored	5

How it will be processed	5

Security	5

How to secure stored data	5

Physical security	5

The UI, User and User Testing	5

The UI	5

The User	6

User Testing	6

Versioning	6

# Introduction

Our IoT application is an alarm system for single-entry rooms containing valuable documents of some form. After passing fingerprint and facial recognition, the electric strike door lock is unlocked, and entry is granted. There is a door sensor to detect when the door is open and closed. A Database is used to store fingerprint, facial images, date, and time of access. Each stage of the process is accompanied by a speaker, buzzer, and LCD display to guide the user through the system. A flask website is to be used to facilitate a typical front-end user.

Team Members Roles:

David Campion: Frontend + UX

Michael Flynn: Database + Testing

Nathan Field: Backend + Testing

Nikita Fedans: Backend + Architecture

# Hardware

## Circuit Diagrams

Circuit:

Fritzing circuit commentary:

- Raspberry pi 3 used for diagram. (4 was unavailable)

- Assuming LEDs need 2.2 forward voltage, the resistor needed is a 68.75 ohm one.

- The grounds of all the LEDs may be connected, and then plugged into a single ground pin on the pi, freeing up n-1 ground pins.

- Momentary switch button will be used for sending a signal to raspberry pi.

- No physical pull down resistor is needed for the switch since an internal pull down resistor may be assigned to a pin through code.

- LCD display requires a large number of wires and specific part was unavailable in fritzing.

- On the PIR sensor in the fritzing diagram, the VCC, OUT, and GND are inverted compared to the PIR sensors bought from pihut, so exercise caution when wiring the real thing.

Problems to solve:

- Research how to restrict the “Field of vision” of the pir sensor to only see a specific area.

- Build homemade door sensor.

- Cross-communication of 2 raspberry pi’s for each side of a doorway.

## Hardware Requirements:

Acquired:

1x pir sensor

1x pi camera module (specially made, plugs into special port)

1x buzzer (works with GPIO and 220-ohm resistor)

1x speaker (hdmx, connected to aux, perhaps powered through USB)

Purchases required:

1x fingerprint sensor https://thepihut.com/products/round-all-in-one-capacitive-fingerprint-sensor-d?variant=41540357849283

1x USB to UART https://thepihut.com/products/usb-to-uart-module-micro-mini-type-a-or-type-c?variant=41771472584899

1x 1.3” IPS LCD Display https://thepihut.com/collections/raspberry-pi-screens/products/1-3-ips-lcd-display-module-for-raspberry-pi-pico-240x240

1x electric strike lock https://www.amazon.co.uk/gp/product/B0027VB810?psc=1

1x door switch (commercial or built)

1x button

# Data, Data Storage and Data Processing

## What Data is gathered

## How it will be stored

## How it will be processed

# Security

## How to secure stored data

Spoofing – 2-factor biometric authentication makes it difficult to falsify identity when using this system.

Tampering – Only admin have access to configuration of this system. Data changes and errors may be logged on the database.

Repudiation – Actions on this system are logged and tracked.

Information Disclosure – Use of env variables to prevent unauthorized access to database, hashing passwords.

Denial of Service – Firewall

Elevation of privilege – Controlled access on routes

## Physical security

System mounted to wall.

Locking mechanism.

# The UI, User and User Testing

## The UI

Flask website

LCD display

## The User

## User Interaction Flowcharts

- Entering from the outside, available here: https://www.canva.com/design/DAFPBEU1cgY/_33aS6_us3ViciJL2B3JKQ/edit?utm_content=DAFPBEU1cgY&utm_campaign=designshare&utm_medium=link2&utm_source=sharebutton

- Exiting from the inside, available here: https://www.canva.com/design/DAFPB6XopUE/RulFr5eydPou51SDKPa_SQ/edit?utm_content=DAFPB6XopUE&utm_campaign=designshare&utm_medium=link2&utm_source=sharebutton

## User Testing

# Versioning

Version 1: Document Creation – 07/10/2022

Version 2: Introduction, Table of Contents and Hardware requirements – 08/10/2022

Version 3: Updated team roles, hardware requirements, STRIDE, User personas and circuit diagram – 14/10/2022